# Assembly Cheat Sheet: Lab 9-15

This document is prepared for open-book exam revision. It includes: lab question labels, solution summaries, code snippets, and key 8086/8088 concepts.

# Key Concepts & Formulae

Chapter summaries and important formulae for ASM exam review

Video memory (Ch6): segment 0xB800, each cell=2 bytes [ASCII][attribute], offset=(row*80+col)*2, write with STOSW.

String instructions (Ch7): LODSB/W, STOSB/W, MOVSB/W, SCASB/W, CMPSB/W; use CLD for forward, STD for backward.

Interrupts (Ch8): INT pushes FLAGS, CS, IP then jumps to handler via IVT at 0:0000. IRET restores FLAGS, CS, IP.

Common screen setup:

mov ax, 0xB800

mov es, ax

cld

mov ax, row

mov bx, 80

mul bx

add ax, col

shl ax, 1

mov di, ax

Core formulae:

Screen offset = (row * 80 + col) * 2

Character in ES:DI = [ASCII byte][attribute byte]

Use REP STOSW to clear/fill screen fast; CX = 2000 for full screen.

Zero DX before DIV for correct division: xor dx, dx

After STD always restore CLD when switching back to forward string ops.

Practice patterns:

Print string to screen using LODSB/STOSW

Clear screen with REP STOSW

Find string or character with REPNE SCASB

Compare blocks with REPE CMPSB/W

Scroll screen with MOVSW and direction flag management

## Lab9/23065 (1).asm

Solution: memcompare takes pointers to block1 and block2 plus length, loops with SI/DI/CX, and returns 0 if equal or 1 if mismatch.

Code:

[org ; Question: Lab9 Task 1 - compare two memory blocks byte-by-byte.

[org 0x100]

jmp start

block1: db 1, 2, 3, 4, 5

block2: db 1, 2, 3, 4, 5

start:

mov ax, block1

push ax

mov ax, block2

push ax

mov ax, 5

push ax

call memcompare

mov ax, 0x4C00

int 0x21

memcompare:

push bp

mov bp, sp

push si

push di

push cx

mov si, [bp+8]

mov di, [bp+6]

mov cx, [bp+4]

cmp_loop:

mov al, [si]

cmp al, [di]

jne mismatch

inc si

inc di

loop cmp_loop

mov ax, 0

jmp cmp_done

mismatch:

mov ax, 1

cmp_done:

pop cx

pop di

pop si

pop bp

ret 6

## Lab9/23065 (2).asm

Question: Lab9 Task 2 - use a jump to a subroutine with a saved return address.

Solution: pushes the return label address then jumps to do_add, which adds AX and BX and returns to after_add.

Code:

[org 0x100]

start:

mov ax, 10

mov bx, 20

mov cx, after_add

push cx

jmp do_add

after_add:

mov ax, 0x4C00

int 0x21

do_add:

add ax, bx

pop cx

jmp cx

## Lab9/23065 (3).asm

Question: Lab9 Task 3 - search for a bit pattern inside a byte stream and return its position.

Solution: bitsearch scans bits by extracting each bit sequence and compares it with the pattern.

Code:

[org 0x100]

jmp start

data:    db 10101000b

pattern: dw 101b

start:

mov ax, data

push ax

mov ax, 1

push ax

mov ax, 101b

push ax

mov ax, 3

push ax

call bitsearch

mov ax, 0x4C00

int 0x21

bitsearch:

push bp

mov bp, sp

push bx

push cx

push dx

push si

push di

mov si, [bp+10]

mov bx, [bp+8]

mov dx, [bp+6]

mov cx, [bp+4]

xor ah, ah

mov al, bl

mov bl, 8

mul bl

xor bh, bh

mov bl, cl

sub ax, bx

inc ax

mov bx, ax

xor di, di

search_loop:

call extract_bits

cmp ax, dx

je found

inc di

dec bx

jnz search_loop

xor ax, ax

jmp search_done

found:

mov ax, di

inc ax

search_done:

pop di

pop si

pop dx

pop cx

pop bx

pop bp

ret 8

extract_bits:

push bx

push cx

push dx

push si

push di

xor bx, bx

mov ch, cl

xor cl, cl

next_bit:

cmp cl, ch

jge eb_done

mov ax, di

shr ax, 3

mov bx, si

add bx, ax

mov al, [bx]

mov dx, di

and dl, 7

mov dh, 7

sub dh, dl

push cx

mov cl, dh

shr al, cl

pop cx

and al, 1

shl bx, 1

or bl, al

inc di

inc cl

jmp next_bit

eb_done:

mov ax, bx

pop di

pop si

pop dx

pop cx

pop bx

ret

## Lab9/23065 (4).asm

Question: Lab9 Task 4 - merge two sorted arrays into one sorted result array.

Solution: merge_sorted compares words from both arrays and copies the smaller into the destination buffer.

Task 4: Merge two sorted arrays

Code:

[org 0x100]

jmp start

arr1:   dw 1, 3, 5, 7

arr2:   dw 2, 4, 6, 8

res:    times 8 dw 0

start:

mov ax, arr1

push ax

mov ax, arr2

push ax

mov ax, 4

push ax

mov ax, 4

push ax

mov ax, res

push ax

call merge_sorted

mov ax, 0x4C00

int 0x21

merge_sorted:

push bp

mov bp, sp

push ax

push bx

push cx

push dx

push si

push di

mov si, [bp+14]

mov di, [bp+12]

mov cx, [bp+10]

mov dx, [bp+8]

mov bx, [bp+6]

merge_loop:

cmp cx, 0

je copy_arr2

cmp dx, 0

je copy_arr1

mov ax, [si]

cmp ax, [di]

jle take_arr1

take_arr2:

mov ax, [di]

mov [bx], ax

add di, 2

add bx, 2

dec dx

jmp merge_loop

take_arr1:

mov ax, [si]

mov [bx], ax

add si, 2

add bx, 2

dec cx

jmp merge_loop

copy_arr1:

cmp cx, 0

je merge_done

mov ax, [si]

mov [bx], ax

add si, 2

add bx, 2

dec cx

jmp copy_arr1

copy_arr2:

cmp dx, 0

je merge_done

mov ax, [di]

mov [bx], ax

add di, 2

add bx, 2

dec dx

jmp copy_arr2

merge_done:

pop di

pop si

pop dx

pop cx

pop bx

pop ax

pop bp

ret 10

## Lab9/23065 (5).asm

Question: Lab9 Task 5 - swap two memory blocks safely using the stack as temporary storage.

Solution: pushes one block onto the stack, copies the second block over the first, then pops back into the second block.

Task 5: Safe memory swap using stack

Code:

[org 0x100]

jmp start

block1: db 1, 2, 3, 4, 5

block2: db 6, 7, 8, 9, 10

start:

mov ax, block1

push ax

mov ax, block2

push ax

mov ax, 5

push ax

call memswap

mov ax, 0x4C00

int 0x21

memswap:

push bp

mov bp, sp

push ax

push bx

push cx

push si

push di

mov si, [bp+8]

mov di, [bp+6]

mov cx, [bp+4]

push cx

mov bx, cx

push_loop:

mov al, [si + bx - 1]

xor ah, ah

push ax

dec bx

jnz push_loop

pop cx

push cx

mov si, [bp+8]

mov di, [bp+6]

copy_loop:

mov al, [di]

mov [si], al

inc si

inc di

loop copy_loop

pop cx

mov si, [bp+8]

mov di, [bp+6]

pop_loop:

pop ax

mov [di], al

inc di

loop pop_loop

pop di

pop si

pop cx

pop bx

pop ax

pop bp

ret 6

## lab10/10q1.asm

Question: Lab10 Q1 - compute the geometric series sum using repeated multiplication.

Solution: power(base, exponent) multiplies base repeatedly, then geosum uses it to sum terms with coefficients.

Code:

[org 0x0100]

jmp start

power:

push bp

mov bp, sp

push bx

push cx

mov bx, [bp+6]

mov cx, [bp+4]

mov ax, 1

cmp cx, 0

je pw_done

pw_loop:

mul bx

loop pw_loop

pw_done:

pop cx

pop bx

pop bp

push ax

ret 4

geosum:

push bp

mov bp, sp

sub sp, 4

push bx

push cx

push dx

mov cx, [bp+4]

mov word [bp-2], 0

mov word [bp-4], 0

gs_loop:

cmp cx, 0

je gs_done

mov bx, [bp+6]

push bx

push word [bp-2]

call power

pop ax

mov bx, [bp+8]

mul bx

add [bp-4], ax

inc word [bp-2]

dec cx

jmp gs_loop

gs_done:

mov ax, [bp-4]

pop dx

pop cx

pop bx

mov sp, bp

pop bp

push ax

ret 6

start:

mov ax, 2

push ax

mov ax, 3

push ax

mov ax, 4

push ax

call geosum

pop ax

mov ax, 0x4c00

int 0x21

## lab10/10q2.asm

Question: Lab10 Q2 - rotate an 8-bit value left N times and compare the result with the original.

Solution: rotateleft performs the bit rotation, then rotateandcompare returns 1 if rotated value is larger.

Code:

rotateleft:

push bp

mov bp, sp

push cx

mov ax, [bp+6]

mov cx, [bp+4]

cmp cx, 0

je rl_done

rl_loop:

rol ax, 1

loop rl_loop

rl_done:

pop cx

pop bp

push ax

ret 4

rotateandcompare:

push bp

mov bp, sp

push bx

mov bx, [bp+6]

push word [bp+6]

push word [bp+4]

call rotateleft

pop ax

cmp ax, bx

jbe rac_zero

mov ax, 1

jmp rac_done

rac_zero:

mov ax, 0

rac_done:

pop bx

pop bp

push ax

ret 4

start:

mov ax, 0xFF00

push ax

mov ax, 8

push ax

call rotateandcompare

pop ax

## lab10/10q3.asm

Question: Lab10 Q3 - reverse each byte in a word and combine two reversed bytes into DX.

Solution: reverse_byte flips the bit order inside one byte, and mirrorswap calls it for the low and high bytes.

Code:

[org 0x0100]

jmp start

reverse_byte:

push bp

mov bp, sp

push bx

push cx

mov bl, [bp+4]

mov bh, 0

mov cx, 8

rb_loop:

shl bh, 1

shr bl, 1

adc bh, 0

loop rb_loop

mov al, bh

mov ah, 0

pop cx

pop bx

pop bp

push ax

ret 2

mirrorswap:

push bp

mov bp, sp

push bx

push cx

mov ax, [bp+6]

mov al, ah

push ax

call reverse_byte

pop ax

mov dl, al

mov ax, [bp+4]

push ax

call reverse_byte

pop ax

mov dh, al

pop cx

pop bx

pop bp

push dx

ret 4

start:

mov ax, 0xAB12

push ax

mov ax, 0x34CD

push ax

call mirrorswap

pop ax

mov ax, 0x4C00

int 0x21

## lab10/10q4.asm

Question: Lab10 Q4 - mirror the low nibble into the high nibble of a byte.

Solution: nibblemirror extracts the low nibble, reverses its bit order, and writes it into the high nibble.

Code:

[org 0x0100]

jmp start

nibblemirror:

push bp

mov bp, sp

push bx

push cx

mov bx, [bp+4]

mov al, bl

and al, 0x0F

mov ah, 0

mov cx, 4

nm_loop:

shl ah, 1

shr al, 1

adc ah, 0

loop nm_loop

and bl, 0x0F

shl ah, 4

or bl, ah

mov ax, bx

pop cx

pop bx

pop bp

push ax

ret 2

start:

mov ax, 0x0005

push ax

call nibblemirror

pop ax        ; AX = 00A5

mov ax, 0x4C00

int 0x21

## lab10/10q5.asm

Question: Lab10 Q5 - compute the difference between the low bytes of two words and return its absolute value.

Solution: bytediff subtracts the low bytes, uses twoscomp for negative results, and duplicates the final byte into AH.

Code:

[org 0x0100]

jmp start

twoscomp:

push bp

mov bp, sp

mov al, [bp+4]

neg al

mov ah, 0

pop bp

push ax

ret 2

bytediff:

push bp

mov bp, sp

push bx

mov ax, [bp+6]

mov bx, [bp+4]

mov al, ah        ; AL = upper byte of A

sub al, bl        ; AL = AL - lower byte of B

jns bd_positive

push ax           ; pass AL

call twoscomp

pop ax

jmp bd_store

bd_positive:

; AL already correct

bd_store:

mov ah, al        ; duplicate into both bytes

pop bx

pop bp

push ax

ret 4

start:

mov ax, 0x5020

push ax

mov ax, 0x0030

push ax

call bytediff

pop ax            ; AX = 2020

mov ax, 0x4C00

int 0x21

## LAB11/BSCS23065 (1).asm

Question: Lab11 Task 1 - clear the screen and print several colored strings to VGA text mode.

Solution: clearscreen writes spaces to video memory, then printstr prints each string with its attribute color.

Code:

[org 0x0100]

jmp start

line1: db 'Hello, this is me saif '

line2: db 'Assembly language by bao g '

line3: db 'We are writing '

line4: db 'Each char is two bytes on screen'

line5: db 'Attribute byte controls the color of ch'

line6: db 'VGA base address is 0xB800'

len1: dw 24

len2: dw 25

len3: dw 31

len4: dw 32

len5: dw 34

len6: dw 26

clearscreen:

push es

push ax

push cx

push di

mov ax, 0xB800

mov es, ax

mov di, 0

mov cx, 2000

clrloop:

mov word [es:di], 0x0720

add di, 2

loop clrloop

pop di

pop cx

pop ax

pop es

ret

printstr:

push bp

mov bp, sp

push es

push ax

push cx

push si

push di

mov ax, 0xB800

mov es, ax

mov di, [bp+10]

mov si, [bp+6]

mov cx, [bp+4]

mov ah, [bp+8]

nextchar:

mov al, [si]

mov [es:di], ax

add di, 2

add si, 1

loop nextchar

pop di

pop si

pop cx

pop ax

pop es

pop bp

ret 8

start:

call clearscreen

mov ax, 164

push ax

mov ax, 0x0B

push ax

mov ax, line1

push ax

push word [len1]

call printstr

mov ax, 484

push ax

mov ax, 0x0A

push ax

mov ax, line2

push ax

push word [len2]

call printstr

mov ax, 804

push ax

mov ax, 0x0E

push ax

mov ax, line3

push ax

push word [len3]

call printstr

mov ax, 1124

push ax

mov ax, 0x0C

push ax

mov ax, line4

push ax

push word [len4]

call printstr

mov ax, 1444

push ax

mov ax, 0x0D

push ax

mov ax, line5

push ax

push word [len5]

call printstr

mov ax, 1764

push ax

mov ax, 0x0F

push ax

mov ax, line6

push ax

push word [len6]

call printstr

mov ax, 0x4C00

int 0x21

## LAB11/BSCS23065 (2).asm

Question: Lab11 Task 2 - scan screen text and change the color attribute for non-space characters.

Solution: reads bytes from VGA memory and updates the attribute byte to yellow for visible characters.

Code:

[org 0x0100]

start:

mov ax, 0xB800

mov es, ax

mov di, 0

mov bx, 0

scanloop:

mov al, [es:di]     ; read ASCII byte

cmp al, 0x20

je isspace

mov bx, 0           ; reset counter

jmp changecolor

isspace:

inc bx

cmp bx, 3          ; 3 continuous spaces = end of text

je done

changecolor:

mov byte [es:di+1], 0x0E    ; overwrite attribute byte only

add di, 2

jmp scanloop

done:

mov ax, 0x4C00

int 0x21

## LAB11/BSCS23065 (3).asm

Question: Lab11 Task 3 - count the number of spaces on the screen and print the result in hex.

Solution: scans VGA text memory, counts spaces, converts the count to hex digits, and prints them.

Code:

[org 0x0100]

start:

mov ax, 0xB800

mov es, ax

mov di, 0

mov cx, 2000

mov bx, 0

countloop:

mov al, [es:di]     ; read ASCII byte

cmp al, 0x20        ; is it a space?

jne notspace

inc bx

notspace:

add di, 2

loop countloop

mov di, 3840        ; bottom-left position

mov ax, bx          ; number to convert

mov bx, 16          ; base 16

mov cx, 0           ; digit counter

nextdigit:

mov dx, 0           ; clear DX before division

div bx              ; AX=quotient, DX=remainder

cmp dl, 9           ; is remainder <= 9?

jle isnum

add dl, 0x37        ; A-F

jmp push_digit

isnum:

add dl, 0x30        ; 0-9

push_digit:

push dx

inc cx

cmp ax, 0

jnz nextdigit

printloop:

pop dx

mov dh, 0x07        ; white on black attr

mov [es:di], dx     ; print digit

add di, 2

loop printloop

mov ax, 0x4C00

int 0x21

## LAB11/BSCS23065 (4).asm

Question: Lab11 Task 4 - display two reversed text lines on the screen.

Solution: writes the reversed strings character by character to video memory.

Code:

[org 0x0100]

jmp start

line1: db 'enil rehto na si siht'

line2: db 'enil a si siht'

len1: dw 21

len2: dw 14

start:

mov ax, 0xB800

mov es, ax

mov di, 0

mov si, line1

mov cx, [len1]

mov ah, 0x07

l1: mov al, [si]

mov [es:di], ax

add di, 2

add si, 1

loop l1

mov di, 160

mov si, line2

mov cx, [len2]

mov ah, 0x07

l2: mov al, [si]

mov [es:di], ax

add di, 2

add si, 1

loop l2

mov ax, 0x4C00

int 0x21

## LAB11/BSCS23065 (5).asm

Question: Lab11 Task 5 - draw a diamond shape of stars on the screen using rows and spacing.

Solution: uses row and position counters to print top and bottom halves of the diamond with spaces and stars.

Code:

[org 0x0100]

jmp start

length: dw 5

i:      dw 0

row:    dw 0

start:

mov ax, 0xB800

mov es, ax

mov word [i], 0

mov word [row], 0

toploop:

mov ax, [i]

cmp ax, [length]

je bottom

mov ax, [row]

mov bx, 160

mul bx

mov di, ax

mov cx, [length]

sub cx, 1

sub cx, [i]

mov ah, 0x07

mov al, 0x20

topspace:

cmp cx, 0

je topstar

mov [es:di], ax

add di, 2

dec cx

jmp topspace

topstar:

mov cx, [i]

shl cx, 1

inc cx

mov al, '*'

topstarloop:

cmp cx, 0

je topdone

mov [es:di], ax

add di, 2

dec cx

jmp topstarloop

topdone:

inc word [i]

inc word [row]

jmp toploop

bottom:

mov ax, [length]

sub ax, 2

mov [i], ax

botloop:

mov ax, [i]

cmp ax, 0

jl done

mov ax, [row]

mov bx, 160

mul bx

mov di, ax

mov cx, [length]

sub cx, 1

sub cx, [i]

mov ah, 0x07

mov al, 0x20

botspace:

cmp cx, 0

je botstar

mov [es:di], ax

add di, 2

dec cx

jmp botspace

botstar:

mov cx, [i]

shl cx, 1

inc cx

mov al, '*'

botstarloop:

cmp cx, 0

je botdone

mov [es:di], ax

add di, 2

dec cx

jmp botstarloop

botdone:

dec word [i]

inc word [row]

jmp botloop

done:

mov ax, 0x4C00

int 0x21

## LAB12/BSCS23065 (1).asm

Question: Lab12 Task 1 - implement a moving text ticker across the top row of the screen.

Solution: prints a message at a shifting screen column, delays, increments the column, and wraps around.

Code:

[org 0x0100]

jmp start

message: db 'Hello World!'

msglen:  dw 12

col:     dw 0

start:

mov ax, 0xB800

mov es, ax

ticker:

; clear row 0

mov di, 0

mov cx, 80

mov ah, 0x07

mov al, 0x20

clearrow:

mov [es:di], ax

add di, 2

loop clearrow

; calculate chars to print

mov ax, 80

sub ax, [col]

mov bx, [msglen]

cmp ax, bx

jge usefull

mov cx, ax

jmp startprint

usefull:

mov cx, bx

startprint:

mov ax, [col]

shl ax, 1

mov di, ax

mov si, message

mov ah, 0x07

printloop:

cmp cx, 0

je delay

mov al, [si]

mov [es:di], ax

add di, 2

add si, 1

dec cx

jmp printloop

delay:

mov cx, 0x00FF      ; reduced delay

outer:

mov bx, 0x00FF

inner:

dec bx

jnz inner

loop outer

inc word [col]

mov ax, [col]

cmp ax, 80

jl ticker

mov word [col], 0

jmp ticker

## LAB12/BSCS23065 (2).asm

Question: Lab12 Task 2 - animate four asterisks moving inward and outward from the screen corners.

Solution: updates corner positions each frame, reverses direction at bounds, and redraws the asterisks.

draws asterisk at all 4 corner positions

erases asterisk at all 4 corner positions

Code:

[org 0x0100]

jmp start

row:  dw 0      ; current row of top-left corner

col:  dw 0      ; current col of top-left corner

dir:  dw 1      ; 1=inward, 0xFFFF=-1=outward

start:

mov ax, 0xB800

mov es, ax

mainloop:

; check keypress to exit

mov ah, 0x01

int 0x16

jz nodone

mov ax, 0x4C00

int 0x21

nodone:

; erase all 4 old positions

call erase

; update position

mov ax, [dir]

cmp ax, 1

je movein

; moving outward

dec word [row]

dec word [col]

jmp checkbounds

movein:

inc word [row]

inc word [col]

checkbounds:

; check if reached center (12,12)

mov ax, [row]

cmp ax, 12

je reverse

; check if reached corners (0,0)

cmp ax, 0

je reverse

jmp draw

reverse:

neg word [dir]

draw:

call drawall

; delay

mov cx, 0x00FF

outer:

mov bx, 0x00FF

inner:

dec bx

jnz inner

loop outer

jmp mainloop

drawall:

push ax

push bx

push di

mov ah, 0x0E        ; yellow attribute

mov al, '*'

; top-left: (row, col)

mov ax, [row]

mov bx, 80

mul bx

add ax, [col]

shl ax, 1

mov di, ax

mov [es:di], ax

; top-right: (row, 24-col)

mov ax, [row]

mul bx

mov di, ax

mov ax, 24

sub ax, [col]

add di, ax

shl di, 1

mov word [es:di], 0x0E2A

; bottom-left: (24-row, col)

mov ax, 24

sub ax, [row]

mul bx

add ax, [col]

shl ax, 1

mov di, ax

mov word [es:di], 0x0E2A

; bottom-right: (24-row, 24-col)

mov ax, 24

sub ax, [row]

mul bx

mov di, ax

mov ax, 24

sub ax, [col]

add di, ax

shl di, 1

mov word [es:di], 0x0E2A

pop di

pop bx

pop ax

ret

erase:

push ax

push bx

push di

; top-left

mov ax, [row]

mov bx, 80

mul bx

add ax, [col]

shl ax, 1

mov di, ax

mov word [es:di], 0x0720

; top-right

mov ax, [row]

mul bx

mov di, ax

mov ax, 24

sub ax, [col]

add di, ax

shl di, 1

mov word [es:di], 0x0720

; bottom-left

mov ax, 24

sub ax, [row]

mul bx

add ax, [col]

shl ax, 1

mov di, ax

mov word [es:di], 0x0720

; bottom-right

mov ax, 24

sub ax, [row]

mul bx

mov di, ax

mov ax, 24

sub ax, [col]

add di, ax

shl di, 1

mov word [es:di], 0x0720

pop di

pop bx

pop ax

ret

## LAB12/BSCS23065 (3).asm

Question: Lab12 Task 3 - search the screen buffer for the string "line" and highlight matches.

Solution: compares a needle string with screen characters and changes the attribute bytes when found.

Code:

[org 0x0100]

jmp start

needle:    db 'line'      ; string to search for

nlen:      dw 4           ; length of search string

start:

mov ax, 0xB800

mov es, ax

mov di, 0             ; current screen position (byte offset)

mov cx, 2000          ; total screen characters

scanloop:

; compare needle with screen at current position

push cx

push di

mov si, needle

mov cx, [nlen]

mov bx, di            ; save start of match position

matchloop:

mov al, [es:bx]       ; read ASCII from screen

mov dl, [si]          ; read char from needle

cmp al, dl

jne nomatch           ; mismatch, stop comparing

add bx, 2             ; next screen char

add si, 1             ; next needle char

loop matchloop

; full match found — highlight

pop di                ; restore start position

push di

mov cx, [nlen]

mov bx, di

highlightloop:

mov byte [es:bx+1], 0x4E  ; red background yellow text

add bx, 2

loop highlightloop

nomatch:

pop di

pop cx

add di, 2             ; move to next screen position

loop scanloop

mov ax, 0x4C00

int 0x21

## LAB12/BSCS23065 (4).asm

Question: Lab12 Task 4 - copy a block of screen text to a buffer, clear the screen, and center it.

Solution: saves source rows to a buffer, clears video memory, then writes the buffered block centered on screen.

Code:

[org 0x0100]

jmp start

src_top:  dw 1

src_bot:  dw 3

dst_top:  dw 0

start:

mov ax, 0xB800

mov es, ax

; height = src_bot - src_top + 1 = 3

mov ax, [src_bot]

sub ax, [src_top]

inc ax

mov bx, ax              ; BX = 3

; dst_top = (25 - 3) / 2 = 11

mov ax, 25

sub ax, bx

shr ax, 1

mov [dst_top], ax       ; dst_top = 11

; step 1 - save source into buffer

mov ax, [src_top]

mov cx, 160

mul cx

mov si, ax              ; SI = row 1 offset = 160

mov di, buffer

mov ax, bx

mov cx, 80

mul cx

mov cx, ax              ; CX = 3*80 = 240 words

saveloop:

mov ax, [es:si]

mov [di], ax

add si, 2

add di, 2

loop saveloop

; step 2 - clear screen

mov di, 0

mov cx, 2000

mov ah, 0x07

mov al, 0x20

clrloop:

mov [es:di], ax

add di, 2

loop clrloop

; step 3 - write buffer to center (row 11)

mov ax, [dst_top]

mov cx, 160

mul cx

mov di, ax              ; DI = row 11 offset = 1760

mov si, buffer

mov cx, 240             ; 3 rows * 80 chars

writeloop:

mov ax, [si]

mov [es:di], ax

add si, 2

add di, 2

loop writeloop

mov ax, 0x4C00

int 0x21

buffer: times 480 db 0

## LAB12/ (4).asm

Section for LAB12/ (4).asm

Code:

[org 0x0100]

jmp start

msg1: db 'This is a line'

msg2: db 'This is another line'

len1: dw 14

len2: dw 20

start:

mov ax, 0xB800

mov es, ax

; print msg1 at row 1

mov di, 160

mov si, msg1

mov cx, [len1]

mov ah, 0x07

p1: mov al, [si]

mov [es:di], ax

add di, 2

add si, 1

loop p1

; print msg2 at row 3

mov di, 480

mov si, msg2

mov cx, [len2]

mov ah, 0x07

p2: mov al, [si]

mov [es:di], ax

add di, 2

add si, 1

loop p2

mov ax, 0x4C00

int 0x21

## Lab13/23065 (1).asm

Question: Lab13 Task 1 - print colored text using alternating character/color pairs.

Solution: loads string bytes and attribute bytes from data and writes them to VGA text memory.

subroutine: printcolor

receives via stack: address of chardata, count

[bp+4] = count

[bp+6] = address of chardata

Code:

[org 0x0100]

jmp start

chardata: db 'H', 0x0B, 'e', 0x0C, 'l', 0x0D, 'l', 0x0E, 'o', 0x0F

db ' ', 0x07, 'S', 0x0A, 'a', 0x0B, 'i', 0x0C, 'f', 0x0D

db 'i', 0x0E, '!', 0x0F

charcount: dw 12

printcolor:

push bp

mov bp, sp

push si

push di

push ds

push es

mov ax, cs

mov ds, ax

mov ax, [bp+6]

mov si, ax

mov ax, 0xB800

mov es, ax

xor di, di

mov cx, [bp+4]

nextchar:

lodsb

mov ah, [si]

inc si

stosw

loop nextchar

pop es

pop ds

pop di

pop si

pop bp

ret 4

start:

mov ax, cs

mov ds, ax

mov ax, 0xB800

mov es, ax

xor di, di

mov ax, 0x0720

mov cx, 2000

rep stosw

push word chardata

push word [charcount]

call printcolor

mov ax, 0x4C00

int 0x21

## Lab13/23065 (2).asm

Question: Lab13 Task 2 - animate two bouncing asterisks moving across the screen borders.

Solution: updates positions, reverses direction at edges, erases previous positions, and redraws them.

asterisk 1

asterisk 2

Code:

[org 0x0100]

jmp start

r1:     dw 0

c1:     dw 0

rd1:    dw 1        ; row dir down

cd1:    dw 1        ; col dir right

r2:     dw 0

c2:     dw 79

rd2:    dw 1        ; row dir down

cd2:    dw 0xFFFF   ; col dir left (-1)

start:

mov ax, 0xB800

mov es, ax

; clear screen

xor di, di

mov ax, 0x0720

mov cx, 2000

rep stosw

mainloop:

; check keypress to exit

mov ah, 0x01

int 0x16

jz nodone

mov ax, 0x4C00

int 0x21

nodone:

; erase asterisk 1

mov ax, [r1]

mov bx, 80

mul bx

add ax, [c1]

shl ax, 1

mov di, ax

mov word [es:di], 0x0720

; erase asterisk 2

mov ax, [r2]

mov bx, 80

mul bx

add ax, [c2]

shl ax, 1

mov di, ax

mov word [es:di], 0x0720

; update asterisk 1 position

mov ax, [r1]

add ax, [rd1]

mov [r1], ax

mov ax, [c1]

add ax, [cd1]

mov [c1], ax

; bounce asterisk 1

mov ax, [r1]

cmp ax, 24

jl chkr1top

mov word [rd1], 0xFFFF

chkr1top:

cmp ax, 0

jg chkc1right

mov word [rd1], 1

chkc1right:

mov ax, [c1]

cmp ax, 79

jl chkc1left

mov word [cd1], 0xFFFF

chkc1left:

cmp ax, 0

jg ast2update

mov word [cd1], 1

; update asterisk 2 position

ast2update:

mov ax, [r2]

add ax, [rd2]

mov [r2], ax

mov ax, [c2]

add ax, [cd2]

mov [c2], ax

; bounce asterisk 2

mov ax, [r2]

cmp ax, 24

jl chkr2top

mov word [rd2], 0xFFFF

chkr2top:

cmp ax, 0

jg chkc2right

mov word [rd2], 1

chkc2right:

mov ax, [c2]

cmp ax, 79

jl chkc2left

mov word [cd2], 0xFFFF

chkc2left:

cmp ax, 0

jg drawboth

mov word [cd2], 1

drawboth:

; draw asterisk 1

mov ax, [r1]

mov bx, 80

mul bx

add ax, [c1]

shl ax, 1

mov di, ax

mov word [es:di], 0x0E2A

; draw asterisk 2

mov ax, [r2]

mov bx, 80

mul bx

add ax, [c2]

shl ax, 1

mov di, ax

mov word [es:di], 0x0C2A

; delay

mov cx, 0x00FF

outer:

mov bx, 0x00FF

inner:

dec bx

jnz inner

loop outer

jmp mainloop

## Lab13/23065 (3).asm

Question: Lab13 Task 3 - count vowels in a string and display the count in hexadecimal.

Solution: scans the string for vowel characters, increments a counter, converts the count to hex, and prints it.

Code:

[org 0x0100]

jmp start

mystring: db 'Hello saifi This Is Assembly'

strlen:   dw 36

vowelcount: dw 0

start:

mov ax, cs

mov ds, ax

mov ax, 0xB800

mov es, ax

; clear screen

xor di, di

mov ax, 0x0720

mov cx, 2000

rep stosw

; count vowels

mov si, mystring

mov cx, [strlen]

mov word [vowelcount], 0

nextchar:

lodsb               ; AL = next char, SI++

cmp al, 'a'

je isvowel

cmp al, 'e'

je isvowel

cmp al, 'i'

je isvowel

cmp al, 'o'

je isvowel

cmp al, 'u'

je isvowel

cmp al, 'A'

je isvowel

cmp al, 'E'

je isvowel

cmp al, 'I'

je isvowel

cmp al, 'O'

je isvowel

cmp al, 'U'

je isvowel

jmp notvowel

isvowel:

inc word [vowelcount]

notvowel:

loop nextchar

; print vowel count in hex at last row (offset 3840)

mov di, 3840

mov ax, [vowelcount]

mov bx, 16

mov cx, 0

hexloop:

mov dx, 0

div bx

cmp dl, 9

jle isnum

add dl, 0x37

jmp pushdigit

isnum:

add dl, 0x30

pushdigit:

push dx

inc cx

cmp ax, 0

jnz hexloop

printloop:

pop dx

mov dh, 0x0E

mov [es:di], dx

add di, 2

loop printloop

mov ax, 0x4C00

int 0x21

## Lab13/23065 (4).asm

Question: Lab13 Task 4 - draw a moving asterisk around the screen perimeter in a square path.

Solution: uses phase state to move right, down, left, and up, drawing the asterisk step by step.

Code:

[org 0x0100]

jmp start

row:    dw 0

col:    dw 0

phase:  dw 0        ; 0=top 1=right 2=bottom 3=left

start:

mov ax, 0xB800

mov es, ax

xor di, di

mov ax, 0x0720

mov cx, 2000

rep stosw

mainloop:

; check keypress

mov ah, 0x01

int 0x16

jz nokey

mov ax, 0x4C00

int 0x21

nokey:

; erase old position

mov ax, [row]

mov bx, 80

mul bx

add ax, [col]

shl ax, 1

mov di, ax

mov word [es:di], 0x0720

; update position based on phase

mov ax, [phase]

cmp ax, 0

je dotop

cmp ax, 1

je doright

cmp ax, 2

je dobottom

jmp doleft

dotop:

inc word [col]

mov ax, [col]

cmp ax, 79

jl draw

mov word [phase], 1

jmp draw

doright:

inc word [row]

mov ax, [row]

cmp ax, 24

jl draw

mov word [phase], 2

jmp draw

dobottom:

dec word [col]

mov ax, [col]

cmp ax, 0

jg draw

mov word [phase], 3

jmp draw

doleft:

dec word [row]

mov ax, [row]

cmp ax, 0

jg draw

mov word [phase], 0

jmp draw

draw:

; draw asterisk at new position

mov ax, [row]

mov bx, 80

mul bx

add ax, [col]

shl ax, 1

mov di, ax

mov word [es:di], 0x0B2A

; delay

mov cx, 0x00FF

outer:

mov bx, 0x00FF

inner:

dec bx

jnz inner

loop outer

jmp mainloop

## Lab14/23065 (1).asm

Question: Lab14 Task 1 - convert lowercase letters to uppercase and uppercase to lowercase while printing.

Solution: checks ASCII ranges and adjusts letters before writing them to VGA text memory.

Code:

[org 0x100]

jmp start

message: db 'Hello World This Is COAL Lab', 0

start:

mov ax, cs

mov ds, ax

mov ax, 0xb800

mov es, ax

xor di, di

mov si, message

cld

nextchar:

lodsb

cmp al, 0

je done

cmp al, 65

jl store

cmp al, 90

jle toupper

cmp al, 97

jl store

cmp al, 122

jg store

sub al, 32          ; lowercase to uppercase

jmp store

toupper:

add al, 32          ; uppercase to lowercase

store:

mov ah, 0x07        ; white on black attribute

stosw

jmp nextchar

done:

mov ax, 0x4c00

int 0x21

## Lab14/23065 (2).asm

Question: Lab14 Task 2 - display a long paragraph using LODS/STOS string instructions.

Solution: loads bytes from DS:SI and stores them with attributes to ES:DI until the terminating 0 byte.

Code:

[org 0x100]

jmp start

message: db 'This is a paragraph written in assembly language for COAL Lab at ITU Lahore. '

db 'We are learning string instructions including LODS and STOS. '

db 'These instructions help us process blocks of memory efficiently. '

db 'LODS loads a byte from DS:SI into AL and increments SI automatically. '

db 'STOS stores AX into ES:DI which points to video memory and increments DI. '

db 'Together they allow us to read and display characters one by one. '

db 'This paragraph is long enough to cover more than half of the DOS screen. '

db 'Assembly language is low level but very powerful and educational. '

db 'We are students of Information Technology University Lahore Pakistan. '

db 'This is the end of our paragraph for Task 2 of Lab 14 COAL course.', 0

start:

mov ax, cs

mov ds, ax

mov ax, 0xb800

mov es, ax

xor di, di          ; start from top left

mov si, message

cld

nextchar:

lodsb

cmp al, 0

je done

mov ah, 0x07        ; white on black attribute

stosw

jmp nextchar

done:

mov ax, 0x4c00

int 0x21

## Lab14/23065 (3).asm

Question: Lab14 Task 3 - read keyboard characters and echo them to the screen until Enter is pressed.

Solution: waits for a key with INT 16h, then stores each character to video memory until Enter stops the loop.

Code:

[org 0x100]

start:

mov ax, cs

mov ds, ax

mov ax, 0xb800

mov es, ax

xor di, di          ; start from top left

cld

nextkey:

mov ah, 0x00

int 0x16            ; wait for keypress, ASCII in AL

cmp al, 0x0D        ; Enter pressed?

je done

mov ah, 0x07        ; white on black attribute

stosw               ; print character to screen

jmp nextkey

done:

mov ax, 0x4c00

int 0x21

## Lab14/23065 (4).asm

Question: Lab14 Task 4 - detect left/right arrow press/release events and print a two-letter code.

Solution: polls keyboard scancodes from port 60h and prints LP/LR/RP/RR for arrow press/release actions.

prints 2 chars from DS:SI to ES:DI

Code:

[org 0x100]

jmp start

lp: db 'LP'

lr: db 'LR'

rp: db 'RP'

rr: db 'RR'

start:

mov ax, cs

mov ds, ax

mov ax, 0xb800

mov es, ax

xor di, di

cld

poll:

in al, 0x60         ; read scancode from keyboard port

cmp al, 0x4B        ; left arrow press

je printlp

cmp al, 0xCB        ; left arrow release

je printlr

cmp al, 0x4D        ; right arrow press

je printrp

cmp al, 0xCD        ; right arrow release

je printrr

cmp al, 0x01        ; ESC to exit

je done

jmp poll

printlp:

mov si, lp

call printstr

jmp poll

printlr:

mov si, lr

call printstr

jmp poll

printrp:

mov si, rp

call printstr

jmp poll

printrr:

mov si, rr

call printstr

jmp poll

printstr:

lodsb

mov ah, 0x07

stosw

lodsb

mov ah, 0x07

stosw

ret

done:

mov ax, 0x4c00

int 0x21

## lab15/23065.asm

Question: Lab15 Task 1 - implement a simple Tic-Tac-Toe style game with limited markers and winner detection.

Solution: tracks player turns, board state, and uses a queue to keep only the most recent three marks for each player.

Code:

[org 0x100]

jmp start

board:      db 0,0,0,0,0,0,0,0,0    ; 0=empty 1=p1 2=p2

p1queue:    db 0xFF,0xFF,0xFF        ; LRU queue player1

p2queue:    db 0xFF,0xFF,0xFF        ; LRU queue player2

p1count:    db 0

p2count:    db 0

turn:       db 1                     ; 1=player1 2=player2

cellpos:    dw 322,330,338,642,650,658,962,970,978

row2:       db '   |   |   ',0

sep:        db '---+---+---',0

msgp1win:   db 'Player 1 Wins!', 0

msgp2win:   db 'Player 2 Wins!', 0

msgdraw:    db 'Draw!', 0

wins:       db 0,1,2, 3,4,5, 6,7,8

db 0,3,6, 1,4,7, 2,5,8

db 0,4,8, 2,4,6

start:

mov ax, cs

mov ds, ax

mov ax, 0xb800

mov es, ax

cld

call drawboard

gameloop:

; get keypress

mov ah, 0x00

int 0x16

sub al, '0'         ; ASCII to number

cmp al, 1

jl gameloop

cmp al, 9

jg gameloop

mov bl, al

dec bl              ; 0-based index

mov bh, 0

; check if occupied

mov al, [board + bx]

cmp al, 0

jne gameloop

; save cell index in DL

mov dl, bl

mov al, [turn]

cmp al, 1

je p1move

p2move:

mov al, [p2count]

cmp al, 3

jl p2place

; remove oldest p2 mark

xor bh, bh

mov bl, [p2queue]

mov byte [board + bx], 0

mov al, [p2queue+1]

mov [p2queue], al

mov al, [p2queue+2]

mov [p2queue+1], al

p2place:

xor bh, bh

mov bl, dl

mov byte [board + bx], 2

mov al, [p2count]

cmp al, 3

jge p2qfull

xor bh, bh

mov bl, al

mov [p2queue + bx], dl

inc byte [p2count]

jmp afterplace

p2qfull:

mov [p2queue+2], dl

jmp afterplace

p1move:

mov al, [p1count]

cmp al, 3

jl p1place

; remove oldest p1 mark

xor bh, bh

mov bl, [p1queue]

mov byte [board + bx], 0

mov al, [p1queue+1]

mov [p1queue], al

mov al, [p1queue+2]

mov [p1queue+1], al

p1place:

xor bh, bh

mov bl, dl

mov byte [board + bx], 1

mov al, [p1count]

cmp al, 3

jge p1qfull

xor bh, bh

mov bl, al

mov [p1queue + bx], dl

inc byte [p1count]

jmp afterplace

p1qfull:

mov [p1queue+2], dl

afterplace:

call drawboard

call checkwinner    ; exits if winner found

; check draw: all 9 cells filled

mov cx, 9

mov bx, 0

checkdraw:

mov al, [board + bx]

cmp al, 0

je nodraw

inc bx

loop checkdraw

; draw condition

mov di, 1280

mov si, msgdraw

call printrow

mov ah, 0x00

int 0x16

mov ax, 0x4c00

int 0x21

nodraw:

; switch turn

mov al, [turn]

cmp al, 1

je tog2

mov byte [turn], 1

jmp gameloop

tog2:

mov byte [turn], 2

jmp gameloop

drawboard:

push ax

push cx

push si

push di

call clrscr

; show whose turn at row 0

mov di, 0

mov al, [turn]

add al, '0'         ; convert to ASCII

mov ah, 0x0A        ; green

stosw

mov di, 320

mov si, row2

call printrow

mov di, 480

mov si, sep

call printrow

mov di, 640

mov si, row2

call printrow

mov di, 800

mov si, sep

call printrow

mov di, 960

mov si, row2

call printrow

call drawmarks

pop di

pop cx

pop si

pop ax

ret

printrow:

lodsb

cmp al, 0

je printrowdone

mov ah, 0x07

stosw

jmp printrow

printrowdone:

ret

clrscr:

push ax

push cx

push di

xor di, di

mov ax, 0x0720

mov cx, 2000

rep stosw

pop di

pop cx

pop ax

ret

drawmarks:

push ax

push bx

push cx

push si

push di

mov cx, 9

mov bx, 0

nextcell:

mov al, [board + bx]

mov si, bx

shl si, 1

mov di, [cellpos + si]

cmp al, 1

je drawx

cmp al, 2

je drawo

jmp celldone

drawx:

mov ax, 0x0C58      ; red X

mov [es:di], ax

jmp celldone

drawo:

mov ax, 0x0B4F      ; cyan O

mov [es:di], ax

celldone:

inc bx

loop nextcell

pop di

pop si

pop cx

pop bx

pop ax

ret

checkwinner:

push ax

push bx

push cx

push si

mov si, wins

mov cx, 8

checkloop:

xor bh, bh

mov bl, [si]

mov al, [board+bx]

cmp al, 0

je nextline

mov dl, al

mov bl, [si+1]

mov al, [board+bx]

cmp al, dl

jne nextline

mov bl, [si+2]

mov al, [board+bx]

cmp al, dl

jne nextline

; winner found in DL

call drawboard

mov di, 1280

cmp dl, 1

je winp1

mov si, msgp2win

jmp printwin

winp1:

mov si, msgp1win

printwin:

lodsb

cmp al, 0

je winndone

mov ah, 0x0E

stosw

jmp printwin

winndone:

mov ah, 0x00

int 0x16

mov ax, 0x4c00

int 0x21

nextline:

add si, 3

loop checkloop

pop si

pop cx

pop bx

pop ax

ret